Preparation for your driving class:

Drive your car as fast as you can in an easy, safe and affordable way on a racetrack. There are no speed limits and no traffic tickets! Our experienced instructors will guide you though the day. Learn how to handle your car at the limit. You set your own goals, and go as fast as you feel comfortable going.  Whether you’re out for a day of fun at the racetrack, or you are looking to achieve a competition license, or hone your competition skills, we have the right instructor for you.

You drive your own car. Anyone with a street car or a race car is welcome.  All car types from Hondas to Corvettes to Porsches are welcome. All you need is a helmet and a safe car.  You must be 18 (or 15 with both parents’ consent.)

…or rent one of our cars. We have a number of cars to choose from. Rental prices range from $150 per day (standard street cars) to $700 per day for our top of the line race cars (for experienced drivers only).  Please check with us in advance for availability.

Body Damage:

Body damage to cars rarely occurs. However, you are encouraged to check with your insurance company regarding coverage. Most insurance companies cover damages to your car during a high performance driving school. This is not competition racing. If they tell you that it is not covered, ask them for a list of exclusions in your policy and review them.

Requirements for Registration:

A car in safe operating condition (you must submit a tech inspection form)

Roll bars in convertibles

Seat Belts in good working order

Tires with good amounts of tread remaining

State driver’s license

Long sleeve shirt and pants

A helmet rated 95 Snell or higher

Tech Inspection requirements

Technical requirements are common sense checks to make sure that your car is in good working order. Be sure to review the tech inspection checklist before the day of the event to ensure that everything is in order.

Anyone who does not submit tech inspection form will not be allowed on the track. You can inspect the car yourself.

Wheels & Tires

The wheels should be round without any dents or damage to the mounting surface. The tires should be in good condition. The tread should be above the wear bats. No cords or belts should show. Flat tires repaired with plugs should not be used as these could fail under the high stress of track conditions. Hubcaps or beauty rings should be removed. They can come off causing a dangerous situation for you and the other participants.

Steering & Suspension

Check the wheel bearings by grabbing the tire and trying to move the wheel from side to side. There should not be any play or clunking sounds. The front wheels should move when you turn the steering wheel.  There should not be any excess play.

Engine

The engine should not have any leaks that will allow liquids to fall onto the track. These fluids will cause slippery conditions on the track. This includes radiator fluid (antifreeze) which is very slippery. The battery should be secured with two (2) bolts. The battery terminals should be covered to prevent any arcs in case of accidental contact.

Brakes

The brake system should be in good working order with no leaks in the system. The brake lines should not have any cracks.  The brake fluid should be clear and at the maximum level. The pads should have plenty of life left in them because the high speeds on the track will wear them out faster than the street. The brake lights should function properly.

Safety Equipment

Seatbelts must be in good condition. Factory seatbelts are OK. Cars without fixed roofs are required to have roll bars. Natural fibers are recommended for clothing. Minimum allowable clothing consists of jeans, a T-shirt and closed toe shoes. You will be required to wear a helmet when on the track. The minimum rating is SNELL 95. Newer helmets with a SA rating are recommended. Eye protection (face shield, goggles, safety glasses) is required. The more and better the safety equipment you have and use, the safer you will be.

Miscellaneous

There should not be any exposed wires. The car should have a good gas cap that seals. The seats should be bolted in tightly.

Before You Go Out on the Track:

- Take out every loose item in your car.

- Remove your spare tire and jack.

- Take out all floor mats.

- Any more loose stuff? Take it out.

- Apply some wax to your door. When it dries to a haze, leave it there. Take some racer’s tape (colored duct tape) and put your “numbers” on top of the waxed portion.

- Wipe off the rest of the haze.  At the end of the day, you will be able to remove your numbers without doing damage or leaving residue on your paint.

- Clean your back and front windows with glass cleaner.

- Fasten any unused seatbelts. You don’t want any buckles flying around.

- Apply some white shoe polish from the most outward tread block to around the shoulder of your tires.  Check the shoe polish after your first session, you can tell if your tires are rolling over onto the sidewall.

- Most people need to increase the tire pressure from what they normally use on the street. How much will depend on your tires. When you come off the track after a run, don’t use your parking brake. Use a block of wood to keep your car from rolling. That will trap the heat that could cause your rotors to warp.

- Keep the engine running for a couple of minutes with the hood up. This will allow your engine to cool down and help circulate the hot fluids.

Things to bring…

For the Car: glass cleaner, wax, a quart of oil, rags, white shoe polish (applicator bottle type), a battery operated air compressor, a tire gauge, a tarp to put all of the stuff you took out of your car, and some basic tools.

For You: a change of clothes, suntan lotion, drinks, a cooler, folding chairs, a shade canopy, a camera and friends to enjoy the day with.